# BIRD Router Verification Test Plan

## Directed Tests

| Test Name | Purpose | Expected Result | Coverage |
| --- | --- | --- | --- |
| local_basic_test | Check a basic valid local packet | Local output matches payload and CRC | Local traffic, valid local cfg, payload_len == 1 |
| local_multi_byte_test | Check local packet with multiple payload bytes | Local output forwards all payload bytes correctly | Local traffic, payload_len 2..4 |
| local_seq_non1_valid_test | Check local packet with valid non-1 sequence number | Packet is accepted and forwarded correctly | Local traffic, valid local cfg, seq_num |
| local_seq_max_valid_test | Check local packet with maximum valid sequence number | Packet is accepted and forwarded correctly | Local traffic, valid local cfg, seq_num boundary |
| local_payload_len_large_valid_test | Check local packet with large valid payload length | Full local payload is forwarded correctly | Local traffic, payload_len > 4 |
| local_seq_zero_drop_test | Check local packet with SEQ_NUM=0 | Packet is dropped and drop_cnt increments | Local seq zero, drop count |
| local_frag_zero_drop_test | Check local packet with invalid FRAG_NUM=0 | Packet is dropped and no local output is produced | Invalid local frag, drop count |
| local_invalid_frag_test | Check local packet with invalid local fragment number | Packet is dropped | Invalid local frag, drop count |
| payload_len_zero_drop_test | Check local packet with payload_len=0 | Packet is dropped and drop_cnt increments | payload_len == 0, drop count |
| remote_payload_len_zero_drop_test | Check remote packet with payload_len=0 | Packet is dropped and drop_cnt increments | payload_len == 0, remote traffic, drop count |
| remote_single_fragment_test | Check valid remote packet with a single fragment | Remote output matches expected data and CRC | Remote traffic, valid remote cfg, remote single fragment |
| remote_multi_byte_test | Check remote packet with multi-byte payload | Remote output words match expected payload and CRC | Remote traffic, payload_len 2..4 |
| remote_payload_len_large_valid_test | Check remote packet with large valid payload | Remote output matches expected large payload behavior | Remote traffic, payload_len > 4 |
| remote_seq_max_valid_test | Check remote packet with maximum valid sequence number | Packet is accepted and handled correctly | Remote traffic, valid remote cfg, seq_num boundary |
| remote_seq_zero_drop_test | Check remote packet with SEQ_NUM=0 | Packet is dropped and drop_cnt increments | Remote seq zero, drop count |
| remote_frag_zero_drop_test | Check remote packet with FRAG_NUM=0 | Packet is dropped and drop_cnt increments | Remote frag zero, drop count |
| remote_two_fragment_in_order_test | Check two remote fragments arriving in order | Fragments are merged into correct remote output | Remote two fragments in order, valid remote cfg |
| remote_two_fragment_out_of_order_test | Check two remote fragments arriving out of order | DUT behavior is checked against expected fragment ordering | Remote two fragments out of order |
| remote_three_fragment_in_order_test | Check three remote fragments arriving in order | Fragments are merged correctly | Remote three fragments |
| remote_three_fragment_3_1_2_test | Check three remote fragments arriving in 3-1-2 order | DUT behavior is checked for out-of-order fragment merge | Remote three fragments, fragment ordering |
| remote_seq_mismatch_drop_test | Check remote sequence mismatch while packet is incomplete | Packet is dropped or mismatch is reported | Remote seq mismatch drop, drop count |
| remote_seq_mismatch_same_frag_drop_test | Check remote sequence mismatch using same fragment position | Packet is dropped or mismatch is reported | Remote seq mismatch drop |
| remote_new_packet_while_incomplete_drop_test | Check new remote packet arriving while previous packet is incomplete | Previous incomplete packet is dropped and drop_cnt increments | Remote new packet while incomplete drop |
| reserved_bits_drop_test | Check packet with reserved configuration bits set | Packet is dropped and drop_cnt increments | Reserved bits error, drop count |
| reserved_high_bits_drop_test | Check packet with high reserved bits set | Packet is dropped | Reserved bits error, drop count |
| remote_reserved_high_bits_drop_test | Check remote packet with high reserved bits set | Packet is dropped | Reserved bits error, remote traffic, drop count |
| remote_reserved_mid_bits_drop_test | Check remote packet with middle reserved bits set | Packet is dropped | Reserved bits error, remote traffic, drop count |
| drop_count_accumulation_test | Check multiple invalid packets in sequence | drop_cnt accumulates correctly | Drop count observed |
| drop_count_hold_after_valid_test | Check that drop count holds after a valid packet | drop_cnt does not reset incorrectly after valid traffic | Drop count observed, valid traffic after drop |
| local_backpressure_test | Check local output when receiver applies backpressure | Local output data remains correct and stable | Local backpressure |
| remote_backpressure_test | Check remote output when receiver applies backpressure | Remote output data remains correct and stable | Remote backpressure |
| reset_initial_test | Check reset behavior at the start of simulation | Outputs and counters reset to initial values | Reset initial |
| reset_after_drop_test | Check reset behavior after a dropped packet | Drop state is cleared after reset | Reset after drop |
| reset_during_remote_accumulation_test | Check reset while a remote packet is incomplete | Incomplete remote packet is cleared and normal traffic works after reset | Reset during remote accumulation |
| local_remote_switching_test | Check switching between local and remote traffic | Both local and remote paths operate correctly | Local traffic, remote traffic, switching |
| remote_frag2_seq2_next | Check remote next-case fragment/sequence behavior | DUT behavior is checked for this sequence/fragment case | Remote fragment/sequence case |
| remote_frag3_seq4_next | Check remote next-case fragment/sequence behavior | DUT behavior is checked for this sequence/fragment case | Remote fragment/sequence case |
| remote_frag5_seq3_next | Check remote next-case fragment/sequence behavior | DUT behavior is checked for this sequence/fragment case | Remote fragment/sequence case |

## Random Tests

| Test Name | Purpose | Expected Result | Coverage |
| --- | --- | --- | --- |
| random_all_cases_test | Randomly exercise valid and invalid packet combinations | DUT behavior matches scoreboard expectations for generated scenarios | Local traffic, remote traffic, payload length, drop cases |
| Random local cases | Randomly check local packet fields and payloads | Valid local packets pass and invalid local packets drop | Local cfg, payload_len, seq_num, frag_num |
| Random remote cases | Randomly check remote packet fields, fragments, and payloads | Valid remote packets are handled and invalid remote packets drop | Remote cfg, remote seq zero, remote frag zero, payload_len |
| Random drop cases | Randomly generate invalid packets | drop_cnt increments when invalid packets are dropped | Drop count observed, reserved bits, invalid cfg |

## Functional Coverage Summary

| Area | Functional Coverage Bins |
| --- | --- |
| Traffic | Local transactions, remote transactions |
| Fields | seq_num, frag_num, payload_len, payload |
| Payload Length | payload_len == 0, payload_len == 1, payload_len 2..4, payload_len > 4 |
| Local Config | Valid local cfg, local seq zero, invalid local frag |
| Remote Config | Valid remote cfg, remote seq zero, remote frag zero |
| Errors | Reserved bits error, drop count observed |
| Backpressure | Local backpressure, remote backpressure |
| Reset | Reset initial, reset after drop, reset during remote accumulation |
| Remote Merge | Remote single fragment, two fragments in order, two fragments out of order, three fragments |
| Remote Drop | Remote seq mismatch drop, remote new packet while incomplete drop |